# Comprehensive Engineering Analysis Report: Solar Panel Cleaning Robot

# 1. System Parameters & Constants

| Parameter | Symbol | Value |
| --- | --- | --- |
| Total Mass | m | 0.7 kg |
| Gravity Acceleration | g | 9.81 m/s² |
| Inclination Angle | θ | 21° |
| Center of Gravity Height | h | 111 mm (0.111 m) |
| Track Width (Wheelbase) | w | 180 mm (0.180 m) |
| Wheel Radius | r | 65 mm (0.065 m) |
| Motor Efficiency | η | 60% (0.60) |
| Rolling Resistance Coeff | Crr | 0.02 |
| Brush Drag Force | Fbrush | 0.50 N |
| Brush Reaction Torque | τb | 0.05 Nm (estimated) |

# 2. Inclination Plane Force Analysis

When the robot is placed on the solar panel, its gravitational weight (W) is resolved into two distinct perpendicular vector components relative to the 21° inclined plane. This is the foundational free-body diagram analysis.

### Step 2.1: Total Weight

Formula: W = m · g

Calculation: W = 0.7 kg · 9.81 m/s² = 6.867 N

### Step 2.2: Normal Force (N)

Definition: The force pressing the robot perpendicularly into the solar panel glass. It governs traction and rolling resistance.

Formula: N = W · cos(θ)

Calculation: N = 6.867 N · cos(21°) = 6.867 · 0.9336

Result: N = 6.41 N

### Step 2.3: Downslope Force (Fd)

Definition: The force component acting parallel to the surface, pulling the robot down the slope.

Formula: Fd = W · sin(θ)

Calculation: Fd = 6.867 N · sin(21°) = 6.867 · 0.3584

Result: Fd = 2.46 N

# 3. Static Tipping Analysis (Equilibrium)

To prevent the robot from toppling over its downhill wheels while stationary, the Restoring Moment must exceed the Overturning Moment.

### Step 3.1: Restoring Moment (Mr)

Formula: Mr = N · (w / 2)

Calculation: Mr = 6.41 N · (0.180 m / 2) = 6.41 · 0.090

Result: Mr = 0.577 Nm

### Step 3.2: Static Overturning Moment (Mo,stat)

Formula: Mo,stat = Fd · h

Calculation: Mo,stat = 2.46 N · 0.111 m

Result: Mo,stat = 0.273 Nm

### Step 3.3: Static Factor of Safety (FoS_stat)

Formula: FoS_stat = Mr / Mo,stat

Calculation: 0.577 / 0.273

Result: FoS_stat = 2.11 (Safe)

# 4. Dynamic Force and Acceleration Analysis

### Step 4.1: Available Drive Force (Fdrive)

Assuming 4 DC motors, each with a safe continuous rated torque of 0.3 kg-cm (0.0294 Nm).

Total Raw Torque = 4 · 0.0294 = 0.1176 Nm

Effective Torque (τ_eff) = Total Raw Torque · Gear Efficiency (η)

Calculation: τ_eff = 0.1176 · 0.60 = 0.0706 Nm

Formula: Fdrive = τ_eff / r

Calculation: Fdrive = 0.0706 Nm / 0.065 m

Result: Fdrive = 1.086 N

### Step 4.2: Total Resistance (Fres)

Formula: Fres = Rolling Resistance (Frr) + Brush Drag (Fbrush)

Frr = Crr · N = 0.02 · 6.41 N = 0.128 N

Calculation: Fres = 0.128 N + 0.50 N

Result: Fres = 0.628 N

### Step 4.3: Net Force (Fnet) and Acceleration (a)

Formula: Fnet = Fdrive - Fres

Calculation: Fnet = 1.086 N - 0.628 N

Result: Fnet = 0.458 N

Formula: a = Fnet / m

Calculation: a = 0.458 N / 0.7 kg

Result: a = 0.654 m/s²

# 5. Dynamic Tipping Analysis

### Step 5.1: Inertial Moment (Mi)

Formula: Mi = m · a · h

Calculation: Mi = 0.7 kg · 0.654 m/s² · 0.111 m

Result: Mi = 0.051 Nm

### Step 5.2: Dynamic Overturning Moment (Mo,dyn)

Formula: Mo,dyn = Mo,stat + Mi + Brush Torque (τb)

Calculation: Mo,dyn = 0.273 Nm + 0.051 Nm + 0.050 Nm

Result: Mo,dyn = 0.374 Nm

### Step 5.3: Dynamic Factor of Safety (FoS_dyn)

Formula: FoS_dyn = Mr / Mo,dyn

Calculation: FoS_dyn = 0.577 Nm / 0.374 Nm

Result: FoS_dyn = 1.54 (Safe, above 1.5 threshold)

# 6. Traction Failure Analysis (No-Slip Condition)

To ensure the robot travels strictly horizontally without sliding diagonally down the 21° slope, the lateral and longitudinal forces must be countered by static friction.

Formula: Minimum Friction Coefficient (μ_min) = √ (Fdrive² + Fd²) / N

Calculation: μ_min = √ (1.086² + 2.46²) / 6.41

Calculation: μ_min = √ (1.179 + 6.051) / 6.41 = √ (7.23) / 6.41

Calculation: μ_min = 2.688 / 6.41

Result: μ_min = 0.42

Conclusion: As long as the tires provide a friction coefficient of 0.42 or higher (standard rubber provides ~0.60), the robot will not slip down the panel.